SAY WATT ROBOTICS  ·  ROV TEAM  ·  CONTROL INTERFACE TRADE STUDY

Three ways to fly the ROV.

Prepared for Tyler — Control Interface Lead. Three controller families, the top pick in each, and exactly what goes in and what comes back out. Ranked, cheapest first.

| THE PICK | WHY IT WINS | INPUTS — PILOT TO ROV | OUTPUTS — ROV TO PILOT |
| --- | --- | --- | --- |
| PICK 1 CONSOLE GAMEPAD Logitech F310 $19.99 · wired USB Alternate: Xbox Wireless Controller, ~$52, adds rumble. | The default in this sport and in industry. Blue Robotics ships the BlueROV2 expecting one, and Deep Trekker’s commercial handhelds are dual-joystick gamepads in a rugged shell. Every kid already knows the layout, so pilot practice is spent flying instead of learning the box. Wired-only is a feature on a pool deck: nothing to pair, nothing to charge, no 2.4 GHz fight with thirty other teams. | 6 analog axes — left stick (surge / sway), right stick (yaw / pitch), and two analog triggers used as heave up and down. 10 digital buttons plus a 4-way D-pad for tools, lights and camera presets. Rear switch selects XInput (Windows, Linux) or DirectInput (macOS). | None. The F310 has no rumble, no LEDs and no screen — it is input-only. Every piece of pilot feedback (depth, voltage, current, leak alarm, thruster bars) therefore has to live on the topside GUI. Step up to the Xbox pad if we want a rumble channel for the leak alarm. |
| PICK 2 FLIGHT-STYLE JOYSTICK Thrustmaster T.16000M FCS $79.99 · wired USB Ambidextrous base; left-handed pilots are fine. | A gamepad gives two coarse thumbsticks. This gives one hand fine, absolute control of pitch, roll and yaw with a separate throttle for depth — much better for holding a transect in current. Hall-effect sensors mean no center drift after a season of abuse, where potentiometer sticks wear out. Best use for us is a two-station cockpit: stick flies, gamepad runs the manipulator. | 4 analog axes — stick X and Y, twist rudder for yaw, and a base throttle slider for heave. Twist can be locked out if it makes the pilot seasick. 16 buttons and one 8-way hat switch. 16,000 × 16,000 resolution on pitch and roll. | None at this price — no force feedback, no rumble, nothing but base lighting. Same constraint as the gamepad: the screen is the only feedback channel. Budget the GUI work accordingly, because it is doing all the talking. |
| PICK 3 CUSTOM TOPSIDE PANEL Teensy 4.0 USB-HID build ~$25–45 in parts Season-long build. Parallel track, never critical path. | The only one of the three that can talk back. A Teensy enumerates as a real USB joystick with zero driver work, so we wire exactly the controls the mission needs — a dedicated pot for camera tilt, a keyed arm/disarm, a real E-stop. It is also the strongest engineering-presentation story: decision matrix, human-factors rationale, a genuine build-vs-buy section. Build it. Just do not fly it first. | Whatever we wire — analog joystick modules, potentiometers, arcade buttons, toggle switches, rotary encoders. Teensyduino’s Joystick object supports 6+ axes and 32 buttons, and it runs as a composite device: joystick and serial on one cable. | Real ones, and this is the whole point. Status LEDs for armed / depth-hold / leak, a piezo buzzer for alarms, and a small OLED or seven-segment for depth, voltage and current. Those come back over the serial half of the composite device — one USB cable, both directions. |

THE CALL

Buy two F310s this month — $40 total. One flies, one lives in the spares box. A dead controller at Dauphin Island with no backup costs us a mission run, and a spare is the cheapest insurance on the whole vehicle. Hold the T.16000M until we know whether the 2027 manual rewards a dedicated manipulator station, and put the custom panel on the Phase 3 calendar as a parallel-track build after CAD locks.

ONE CODE PATH.  All three are plain USB HID. The same twenty lines of pygame.joystick in topside/controller.py read any of them, so Tyler can swap hardware without touching the pilot loop.

DEADBAND OR CHATTER.  Map anything inside ±0.05 on every axis to zero, or the thrusters will buzz while the sticks sit at rest.

SECTION 4.3.  Tyler runs the comparison himself and writes the decision matrix. He has to explain the axis map and the pick cold to a judge — and that matrix is worth documentation points either way.

SOURCES: LOGITECHG · THRUSTMASTER · MICROSOFT · BLUEROBOTICS · DEEPTREKKER · PJRC     |      SAY WATT ROBOTICS · ROV TEAM · AUG 2026